# JMD — Assumptions and Verification Register

Status key: C = confirmed by the supplied material · A = illustrative design assumption (new) · U = unconfirmed / conflicting · V = requires further development or verification.

Quantities marked “DWG” were measured from the hatched zones in FINAL_8-10-2026.dwg. Drawing units are read as metres: the dimensioned 6.0 m roads and 1.5 m walkways match the S1 p.4 section. All quantities are indicative; they are measured footprints, not survey or GFA.

## 1. Directly supported by the original material

| # | Item | Value / description | Source | Status |
| --- | --- | --- | --- | --- |
| 1.1 | District programme | Showrooms, arena, Collectors’ Club, Wall of Fame museum, simulator hall, off-road experience, lifestyle retail, launch plaza | S1, S2, S5 | C |
| 1.2 | Overall layout | Curved showroom frontage; retail row behind it; arena and outdoor function area at the south tip; club at the hinge; off-road with test-drive loop; service village and dealer storage to the north-east | DWG | C |
| 1.3 | Showroom units | 52 units: 23 standard (≈240–245 m²), 21 premium (≈350–372 m²), 7 flagship (≈386–461 m²), 1 flagship-plus (540 m²). Footprint ≈16,640 m² | DWG | C |
| 1.4 | Commercial shops | 29 units, ≈388–429 m² each, ≈11,530 m² in total | DWG | C |
| 1.5 | Arena | 7 hall bays ≈12,911 m² plus a concourse band ≈5,911 m² | DWG | C |
| 1.6 | Arena divisibility and modes | Independent or combined halls; 360° / 270° / 180° / runway / exhibition / e-karting | S1 p.17–23 | C (intent) |
| 1.7 | Collectors’ Club | 30 × 117 m footprint (≈3,508 m²) | DWG | C |
| 1.8 | Off-road | Zone incl. loop ≈54,273 m²; inner terrain ≈41,422 m²; 7 obstacle areas ≈17,692 m² | DWG | C |
| 1.9 | Test-drive centre | Registration building ≈2,487 m², drop-off ≈733 m², test-car parking ≈4,232 m², guest parking ≈2,258 m² | DWG | C |
| 1.10 | Service village | 58 light-service and spare-parts units (≈635 m² typical), ≈38,146 m² | DWG | C |
| 1.11 | Dealer storage | ≈55,710 m² plus a 3,926 m² strip (shown as a PDI shed, see 2.10) | DWG | C |
| 1.12 | Other | Energy hub ≈3,793 m²; management ≈1,524 m²; 2 mosques (≈1,092 m² and ≈3,809 m²); commercial block ≈3,809 m²; outdoor function area ≈2,848 m² | DWG | C |
| 1.13 | Visitor parking | 22 single-row bays about 6 m wide, ≈58,444 m² in total, arranged back-to-back with 6 m aisles | DWG, S1 p.4 | C (layout) |
| 1.14 | Frontage identity | Showrooms face the main road with front customer parking (narration: “showroom from the street level … on Madinah Road, and behind it … the arena and the collectors club”) | S2, S1 p.3 | C (intent) |

## 2. Illustrative design assumptions (new in this presentation)

| # | Assumption | Reason | Status |
| --- | --- | --- | --- |
| 2.1 | Building heights: showrooms 9–14 m; shops 8 m; service 9 m; arena 21 m plus roof zone; club 15.5 m (3 levels); mosques 9/12 m with minarets of 28/34 m | Credible massing for concept renders | A |
| 2.2 | Architectural language, materials and façade kit-of-parts (Visual Design Bible) | One coherent district | A |
| 2.3 | Showroom brand bands left blank; no brand logos shown | Avoid implying tenant commitments | A |
| 2.4 | Arena architecture: floating roof over the full triangle, glazed concourse, bronze fins, entrance portal on the plaza | DWG gives the footprint only | A |
| 2.5 | Retail arcade 5.5 m deep on bronze columns | Shade and pedestrian comfort | A |
| 2.6 | Off-road obstacles: mounds ≈10 m, dune field, water crossing, rock crawl, timber ramps, side-slope. Kerbed loop | DWG gives the zones, not their nature | A |
| 2.7 | Launch plaza shade sails and reveal stage | Event use | A |
| 2.8 | Frontage road cross-section (≈76 m, 2 × 4 lanes, service roads, median); south-east road 2 × 3 lanes; local roads on the north and east edges | The DWG shows the site boundary only | A |
| 2.9 | Surrounding context: anonymous low-rise massing north-west of the frontage road; open land to the south-east | Readable urban setting without depicting actual neighbours | A |
| 2.10 | The ZIGZAG strip inside dealer storage is shown as a vehicle-preparation (PDI) shed | Unlabelled in the DWG; the earlier masterplan has a “PDI area” | A / V |
| 2.11 | 14 identical 3.8 × 20.4 m hatches inside shop units are omitted from the massing | Unlabelled; purpose unknown | A / V |
| 2.12 | Interiors (arena modes, museum, simulator hall, club lounge) | Atmosphere only; no interior layouts exist | A |
| 2.13 | Landscape: date palms on road verges and medians, parking edges, promenade and plaza; shrub beds; 3.5 m footways | Climate-appropriate public realm | A |
| 2.14 | Parked vehicles, figures and traffic placed by rule (stall geometry, occupancy 55–88%) | Visual realism only. Not a parking capacity | A |
| 2.15 | Visitor journeys on slide 15 | Indicative only | A |

## 3. Unconfirmed or conflicting site and technical information

| # | Item | Conflict | Status |
| --- | --- | --- | --- |
| 3.1 | Which site the DWG layout relates to | The DWG has no coordinates or site name. S1 shows three location options (p.24–27) and an earlier 180,000 m² layout (p.29–30) | U |
| 3.2 | Site area | 170,000 m² (S5) / 180,000 m² (S1 p.30) vs. the DWG boundary of ≈410,000 m² (41.0 ha) | U |
| 3.3 | Frontage road name and length | “Madinah Road, 1,668 m” (S5); 560 m (S1 p.31–33); DWG frontage arc ≈1,035 m | U |
| 3.4 | Showroom count | 18–20 brands (S5); 41 front + 28 back (S1 p.32); 52 units (DWG) | U |
| 3.5 | Arena form | Glass oval dome (S1 p.2–3), faceted pavilion (p.17) and glass hall (p.18–23) vs. the DWG triangle | U, resolved in the deck to the DWG |
| 3.6 | Arena capacity | 10,000 (S5); per-mode ranges of 4,500–12,000 (S1 p.18–23); floor areas of 12,000–20,000 m² (S1) vs. 12,911 m² of hall bays (DWG) | U |
| 3.7 | Parking quantities | ≈3,000 spaces (S5) vs. ≈5.8 ha of visitor bays plus ≈5.6 ha of storage (DWG) | U |
| 3.8 | Test-drive loop length | 2.5 km (S5) vs. a shorter loop around the off-road zone in the DWG | U |
| 3.9 | Basement parking or storage | Shown in S1 p.4 and S5 (P1); not in the DWG | U |
| 3.10 | Museum and simulator hall | Not located in the DWG; two inconsistent museum exteriors in S1 | U |

## 4. Items requiring further development or verification

| # | Item | Suggested owner / next step |
| --- | --- | --- |
| 4.1 | Confirm the preferred site, its boundary and area; re-fit the layout | Client / planning |
| 4.2 | Traffic impact study: access points, junctions, event-day traffic | Transport consultant with the Municipality |
| 4.3 | Parking demand and provision (daily and event) | Transport consultant |
| 4.4 | Land-use and zoning compatibility | Planning, with the Municipality |
| 4.5 | Utilities: power (EV charging, arena), water, drainage, telecoms | MEP / infrastructure |
| 4.6 | Arena capacity, egress, fire strategy (S1 cites NFPA, unverified) | Venue and fire consultants |
| 4.7 | Off-road safety, dust and noise near public areas | Operator / environmental |
| 4.8 | Locate and size the museum and simulator hall | Design team |
| 4.9 | Building heights, GFA and floor areas (renders are not area statements) | Design team |
| 4.10 | Brand and tenant strategy; signage controls | Client / leasing |
| 4.11 | Pedestrian crossings, shade standards and public-realm specification | Urban design with the Municipality |
| 4.12 | Phasing | Client |

## 5. What the visualisations are and are not

- They are path-traced (Blender Cycles) renders of a 3D model built directly from the DWG footprints. Every view shows the same model, so geometry is consistent across images.

- They are concept visualisations. Vehicles, people and palms are simplified procedural models, and context massing is generic. They are not photographic marketing renders and not technical drawings.

- No AI image generation was used. No real brand marks appear in the new renders. Brand marks appear only in reproduced source pages, which are labelled as source material.

- Separation: no AlBalad Development Company (BDC) material or assumptions were used.